____/____/26 - Y12 Networks L4: The TCP/IP stack and protocol layering

| Today's Topic(s) & Link(s): - To describe the four layers of the TCP/IP stack and explain what each layer adds to the data - PMT 1.3.3 Networks notes (PDF) - OCR Topic Exploration Pack: Networks (PDF) - Wireshark packet analyser (free) - neverssl.com (a site that stays on plain HTTP, ideal for capturing) | Keywords: - TCP/IP stack - application layer - transport layer - internet layer - link layer - packet - frame - header - port - socket - MAC address - IP address - encapsulation | Homework: - See your teacher |
| --- | --- | --- |

Important Points:

★

★

★

Lesson Notes:

Include your own notes here

Lesson Tasks

Starter: complete your SmartRevise 5-A-DAY. Work in OneNote: type your answers in the boxes.

Task 1: Layers from memory (5 min)

Write the four layers of the TCP/IP stack from top to bottom. For each, give one job it does.

| Layer | One job |
| --- | --- |
| 1 (top) | This layer ... |
| 2 |  |
| 3 |  |
| 4 (bottom) |  |

Task 2: The envelope role play (10 min)

Watch the role play. Four pupils each play a layer. Fill in what each layer adds to the envelope.

| Layer | What it adds | Example |
| --- | --- | --- |
| Application | It adds ... |  |
| Transport |  |  |
| Internet |  |  |
| Link |  |  |

a) Why does the receiving computer open the layers in the opposite order?

Your answer

The receiver opens them in reverse because ...

Task 3: Capture a real request (20 min)

First, press the Windows key, type cmd, press Enter, then type ipconfig /all. Screenshot it (Windows key + Shift + S), paste it into OneNote and circle your IPv4 address and physical address (MAC). You will look for these in the capture. On Windows the command is ipconfig, not ifconfig.

If your teacher has given you the capture file fallback-http.pcap, open it in Wireshark instead and start at step 3 (type http in the filter bar).

- Open Wireshark and start capturing on your network connection.

- In a browser visit http://neverssl.com (this site stays on plain HTTP so you can read the packets).

- Stop the capture and type http in the filter bar.

- Click a packet that says GET. In the middle pane expand Ethernet II, Internet Protocol, Transmission Control Protocol and Hypertext Transfer Protocol.

| Field I found | Its value | Which layer? |
| --- | --- | --- |
| Source MAC address |  |  |
| Destination MAC address |  |  |
| Source IP address |  |  |
| Destination IP address |  |  |
| Time to live |  |  |
| Source and destination port |  |  |
| HTTP method and path |  |  |

a) The destination MAC address is not the web server's. Whose is it, and why?

Your answer

The destination MAC address belongs to ... because ...

Task 4: Python: build the stack (15 min)

Open layers_starter.py. Each layer wraps the data in a dictionary with a header. Complete TODO 1 to TODO 6 in order, running the file after each one.

| TODO | What it does | Works? |
| --- | --- | --- |
| 1 | application_layer |  |
| 2 | transport_layer |  |
| 3 | network_layer |  |
| 4 | link_layer |  |
| 5 | decapsulate |  |
| 6 | receive |  |

a) What does your checksum catch, and what does the receiver do when it finds a problem?

Your answer

The checksum catches ... When the receiver finds a problem it ...

Stretch: add a time to live that goes down by one each time a packet passes a router, and drop the packet at zero.

Task 5: Exam practice (10 min)

The protocol TCP/IP uses a 4-layer stack.

Complete the table below to show the 4 layers in the TCP/IP stack.  [2]

Real OCR question. Source: OCR H046/01 June 2022, Q3 (PMT Set B Q5(b)(i))

Layer

Application

Link

Your answer

Explain one advantage of using layers in the protocol TCP/IP.  [2]

Real OCR question. Source: OCR H046/01 June 2022, Q3 (PMT Set B Q5(b)(ii))

Your answer

One advantage is ... This is because ...

Explain why protocol layering is used.  [3]

Real OCR question. Source: OCR H446/01 June 2023, Q1(d)(iv) (PMT Set B Q2(iv))

Your answer

Protocol layering is used because ...

The TCP/IP stack allows communication over large distances such as the internet.

Describe the purpose of the transport layer in the TCP/IP stack.  [2]

Real OCR question. Source: OCR H046/01 June 2025, Q9(b)

Your answer

The transport layer ...

Discuss (Craig and Dave): Explain why a computer has both a MAC address and an IP address.

A MAC address is used for ... An IP address is used for ...

Summary

After the lesson, use this space to summarise the main points of this lesson topic.

Draw the four layers and one thing each adds. Explain MAC versus IP in two sentences.